Design and Implementation
	The proposed system is a compact temperature monitoring and alert device designed to measure temperature continuously and display the measured value through a small OLED screen. The system uses an Arduino Nano as the main microcontroller, an MCP9808 temperature sensor for accurate temperature measurement, and a 0.96-inch OLED display for presenting the temperature reading.

The system is powered through a USB male connector, allowing it to receive power from a computer, power bank, USB adapter, or other compatible USB power source. The breadboard and jumper wires are used during the prototype stage to establish electrical connections between the components.

The basic operation of the system is:

USB Power → Arduino Nano → MCP9808 Temperature Sensor → Temperature Processing → OLED Display

The Arduino Nano receives temperature data from the MCP9808 through the I²C communication interface. After receiving the temperature reading, the Arduino processes the data and sends the result to the OLED display. If the measured temperature reaches a predefined alert threshold, the system can indicate that the temperature has exceeded the desired level.

Hardware Components

| Components | Function |
| --- | --- |
| Arduino Nano | Main microcontroller that processes sensor data and controls the OLED display |
| MCP9808 | Digital temperature sensor used to measure the surrounding or monitored temperature |
| 0.96inch oled | Displays the measured temperature and system status |
| USB male connector | Provides power to the system through a USB connection |
| Breadboard | Used for assembling and testing the prototype circuit |
| Jumper Wires | Used to establish electrical connections between components |

Hardware Designs
	The Arduino Nano serves as the central component of the system. The MCP9808 is connected to the Arduino Nano using the I²C communication protocol, which allows the microcontroller to retrieve temperature measurements from the sensor.

The OLED display also communicates with the Arduino Nano through I²C. Because both the MCP9808 and OLED can operate through the same I²C bus, they can share the communication lines while using their respective device addresses.

  USB Male Connector → Arduino Nano power input

  Arduino Nano → MCP9808

- VCC → 5V/appropriate supply

- GND → GND

- SDA → A4

- SCL → A5

  Arduino Nano → 0.96-inch OLED

- VCC → appropriate supply

- GND → GND

- SDA → A4

- SCL → A5